Assessment Schedule – 2008

Physics: Demonstrate understanding of electricity and electromagnetism (90257)

Evidence Statement

| Q | Evidence | Achievement | Achievement with Merit | Achievement with Excellence |
| --- | --- | --- | --- | --- |
| ONE (a) |  | 1 Downward line. | 1 Evenly spaced parallel lines with curved end(s). |  |
| (b) | - + | 1 Curves towards negative plate. |  |  |
| (c) | The electric force is at right angles to the direction in which the positive particle is moving. This causes the particle to describe a parabolic path. | 1 Force is down. or 1 Repelled from +. or 1 Attracted to –. | 1 Achievement plus has link to forward motion or constant downwards force and parabolic path |  |
| (d) | Magnetic field into the page. | 1 Correct answer. |  |  |
| (e) | The electric force depends only on the electric field strength and the size of the charge. Hence is not affected by the velocity of the particle. The magnetic force F = Bqv increases as the velocity of the particle increases, as the magnetic force is directly proportional to the velocity, provided the magnetic field strength is a constant. | 1Electric force is not affected by the velocity or Magnetic force increases as velocity increases. | 1Electric force is not affected by the velocity, but the magnetic force increases as the velocity increases. | 1Merit, plus F depends only on E and q; eg F=Eq and F = Bqv depends on v. |
| (f) |  | 2Correct formula used to find E, but did not convert cm to m.(E=44 ) | 2Correct value for E (4400) or F using cm 2 F = 7.0  10–18 N | 2Correct answer. 7.0  10–18 |
|  |  | 2 Correct sig figs. Any attempt to find F correct to 2sf |  |  |
| (g) |  | 2 Correct except for charge, eg | 2 Correct answer. 5.6  10–5 A |  |
| TWO (a) | 12 J | Two grades here 2 Correct number. 2 Correct unit. |  |  |
| (b) | This is a “show” question: = 4.5 + 2.06 = 6.56  | 2States | This is a “show” question: 2 Correct working. |  |
| (c) |  | 2Correct answer. |  |  |
| (d) | Effective resistance of 3.4  and 5.2  = 2.06 and V=IR=1.83  2.06 = 3.8 V OR Voltage across 4.5  resistor is: V = IR = 8.235 V Voltage across the 3.4  resistor is 12 – 8.235 = 3.8 V |  | 2Calculates voltage correctly. 3.8 V |  |
| (e) | The voltage across the 5.2  resistor will also be 3.8 V, as it is in parallel with the 3.4  resistor. | 1Mentions that the voltage is 3.8 V. | 1Achievement, plus states that this is because it is in parallel with the 3.4  resistor. |  |
| (f) | The circuit current depends on the supply voltage and the effective resistance. The supply voltage does not change. Current is inversely proportional to resistance. The diode is in the reverse bias and hence no current flows through the 5.2  resistor. The effective resistance of the circuit increases to 7.9 . This means the circuit current will decrease. | 1Mentions that current will decrease. or Resistance increases. | 1Achievement, plus states that the current decreases because the resistance increases | 1Merit, plus gives reason why the resistance increases (removing another path), or states that the supply voltage does not change. Or R increases to 7.9 |
| (g) | OR Could also use P = I2 R= 1.522  3.4 | 2Calculated new current (1.52 A) | 2Calculated voltage across the 3.4  resistor correctly, but did not convert minute to seconds for calculating energy and gets P = 5.168  1.52 = 7.8 | 2Correct answer. 471 J |
| THREE (a) |  | 2Correct except for one mistake. Either incorrect unit conversion (4.4) or missing x2 (0.022) | 2 Correct answer. 0.044 |  |
| (b) | Stronger magnetic field, longer length of wire in the field, increasing the speed with which the wire is made to move in the magnetic field. | 1Correct answer. |  |  |
| (c) | The induced voltage depends on the speed that the wire cuts across the magnetic field. The maximum induced voltage is produced when the coil is moving perpendicular to the direction of the magnetic field. The size of the induced voltage is minimum when the coil is moving parallel to the direction of the magnetic field. Hence the induced voltage changes from a maximum to a minimum every quarter cycle. | 1Max when coil is flat or moving at 90 deg to field or Min when coil is upright/vertical at 90 deg to field or moving along field. | 1 States the condition for both maximum and minimum voltage to be induced. | 1 Full and correct explanation eg linking changing velocity perpendicular to the field to voltage. or Voltage must change from max to min (must state when min and max occur) |
| (d) |  | 2 Correct current. 2.66 A | 2Correct answer except for number of turns. or Unit conversion (2800 N) | 2Correct answer. 28N (29 N) |

Judgement Statement – 2008

| Achievement | Achievement with Merit | Achievement with Excellence |
| --- | --- | --- |
| 5  A1 | 4  A1 + 3  M1 | 4  A1 + 2  M1 + 2  E1 |
| 5  A2 | 4  A2 + 3  M2 | 4  A2 + 2  M2 + 2  E2 |