Assessment Schedule – 2009

Physics: Demonstrate understanding of electricity and electromagnetism (90257)

Evidence Statement

| Q | Evidence | Achievement | Merit | Excellence |
| --- | --- | --- | --- | --- |
| ONE (a) | Arrow towards top of page. | 1 Correct answer. |  |  |
| (b) |  | 2 Correct charge. | 2 Correct answer. |  |
| (c) | The force remains constant because the electric field strength is constant (F = Eq). | . | 1 Correct answer |  |
| (d) | If the voltage increases, the force on the ion increases. Greater force means greater acceleration, which means greater maximum velocity. OR If the voltage increases, the ion gains more kinetic energy, and therefore has a greater velocity. | 1 Greater velocity. Except if based on V=Bvl, | 1 Achievement plus partial explanation. | 1 Correct answer and full and concise explanation. |
| (e) |  | 2 Correct answer. |  |  |
| (f) |  | 2 Correct KE. | 2 Correct working except for one error. | 2 Correct working and answer. |
| TWO (a) |  | 2 Correct answer. |  |  |
| (b) |  | 2 One correct step. Eg 10 ohms or 4 from parallel. | 2 Correct answer except for one error. Eg 1.2 A | 2 Correct answer. |
| (c) | If the resistance increases, the total resistance increases and the total current decreases. This means the voltage across the two series resistors decreases. | 1 One correct idea. Must answer question correctly. | 1 Two correct ideas. | 1 Correct answer and explanation. |
| (d) | A diode only allows current to flow in one direction. | 1 Correct answer. |  |  |
| THREE (a) | To the right. | 1 Correct answer. |  |  |
| (b) |  | 2 Correct current. | 2 Correct answer except for one error. Eg no unit. | 2 Correct answer. |
| (c) | No. The two wires carry current in opposite direction. The force on the two wires is in the opposite direction. The forces are equal and so they cancel. | 1 No force. Except if because yachts are stationary. | 1 Currents are equal and opposite. | 1 No. Currents opposite, forces equal and opposite. |
| (d) | Yes there is a voltage induced because the two wires are cutting across a magnetic field. This causes an induced voltage. |  | 1 Correct answer. Must convey movement at 90 deg to /across field. |  |
| (e) |  | 2 Correct answer. |  |  |

Judgement Statement

| ### Achievement | ### Achievement with Merit | ### Achievement with Excellence |
| --- | --- | --- |
| 3 A1 2 A2 + another A | 2 A1 + 2 M1 2 A2 + 2 M2 | 2 A1 + 2 M1 + 1 E1 2 A2 + 2 M2 + 1 E2 |